NETWORKING
PROJECT

HOME NETWORK DESIGN, CONFIGURATION
AND CONNECTIVITY TESTING

A Cisco Packet Tracer simulation

| PROJECT DETAILS | INFORMATION |
| --- | --- |
| Student Name | Waqas Hussain - GlitchWH |
| Registration / Roll Number | ****CSE2418 |
| Program / Department | Computer System Engineering |
| Course | Computer Communication and Networking (CCN) |
| Instructor | Prof Dr Ihsan Ul Haq |
| Institution | UET Peshawar |
| Software | Cisco Packet Tracer |
| Date | 10/11/2026 |

Prepared as a technical record of the simulated home-network project

# Executive Summary

This project documents the design and configuration of a simulated home network in Cisco Packet Tracer. The intended network provides wired connectivity for two desktop computers, wireless connectivity for a laptop, and a separate coaxial connection for a television. A cable splitter separates the incoming cable signal for the cable modem and TV. The modem connects to the Internet port of a home wireless router, which provides local wired ports, Wi-Fi, and DHCP services.

The router was configured with a limited DHCP client pool, a changed administrator password, and a 2.4 GHz wireless network named MyHome secured with WPA2 Personal. During troubleshooting, clients could reach the router gateway, but the hostname skillsforall.srv initially could not be resolved. A DNS server was then added and configured in the simulation, after which the browser displayed a Cisco Packet Tracer welcome page at that hostname. The exact DNS server address and complete final WAN configuration were not recorded, so those values are intentionally left for verification rather than assumed.

Project outcome The hostname skillsforall.srv eventually opened a webpage showing “Welcome to Cisco Packet Tracer — Opening Opportunities, Mind Wide Open” and Quick Links. This confirmed that the browser reached a page for the configured hostname in the simulation. Verify all three clients and record the final DNS/WAN values before submitting this report.

# Table of Contents

1. Introduction

2. Objectives and Scope

3. Tools and Network Components

4. Network Design and Topology

5. Addressing and Configuration Plan

6. Implementation Procedure

7. Troubleshooting and Resolution

8. Testing and Verification

9. Security Considerations

10. Results and Discussion

11. Conclusion and Future Improvements

Appendix A — Configuration Checklist

Appendix B — Key Networking Concepts

# 1. Introduction

A home network allows multiple devices to share local resources and access external services through a common connection. In this practical project, a simulated cable-provider connection is distributed to internet and television devices. A home wireless router acts as the central local-network device, connecting wired computers and a Wi-Fi laptop.

The work was performed in Cisco Packet Tracer, which models network devices, links, IP configuration, and selected services. Because this is a simulation, results describe the Packet Tracer environment and should not be interpreted as proof of a real internet connection from the student's physical laptop.

# 2. Objectives and Scope

## 2.1 Objectives

Connect the coaxial cable splitter to the cable modem and television.

Connect the cable modem to the router's Internet/WAN port.

Connect two desktop PCs to the router's LAN Ethernet ports.

Configure the router's DHCP pool and administrator password.

Enable a 2.4 GHz WLAN named MyHome and secure it with WPA2 Personal.

Connect a laptop to the WLAN using the configured passphrase.

Configure or verify DNS so the hostname skillsforall.srv can be resolved in the simulation.

Test client connectivity and document troubleshooting results.

## 2.2 Scope

The report covers device placement/identification, physical and logical connections, router configuration, Wi-Fi security, client IP configuration, DNS troubleshooting, and browser-based connectivity tests. It does not claim that advanced routing, VLANs, firewall policies, or a production-grade internet uplink were configured.

# 3. Tools and Network Components

| Component | Role in the project |
| --- | --- |
| Cisco Packet Tracer | Simulation environment used to assemble and test the network. |
| Cable Splitter | Splits the incoming coaxial signal into separate outputs for internet and television. |
| Cable Modem | Terminates the cable-provider connection and presents an Ethernet connection to the router. |
| Home Wireless Router | Provides WAN connectivity, local Ethernet switching, Wi-Fi, and DHCP for client devices. |
| Office PC | Wired client used to access the router GUI and test the website. |
| Bedroom PC | Second wired client; configured to obtain IP settings through DHCP. |
| Laptop | Wireless client that joins the MyHome SSID. |
| Television | Receives the television signal through the splitter's second coaxial output. |
| DNS server (added during troubleshooting) | Resolves the hostname skillsforall.srv to the configured web-server address in the simulation. |

# 4. Network Design and Topology

The logical design follows the path below. The laptop uses Wi-Fi; the desktop computers use Ethernet; the TV uses coaxial cable.

Cable Provider / Incoming Coaxial Signal
                 │
           Cable Splitter
            ┌────┴────┐
            │         │
       Cable Modem   Television
            │ Ethernet
            ▼
     Home Wireless Router
       ┌────┼─────┐
       │    │     ))) Wi-Fi
    Office  Bedroom   Laptop
      PC      PC

Topology note A DNS server was added during troubleshooting because the hostname was unresolved and the router/client DNS field was initially 0.0.0.0. Its exact placement, address, and web-service configuration were not captured in the available record; add those details from the final Packet Tracer file.

## 4.1 Physical links

| Source device / port | Destination device / port | Medium |
| --- | --- | --- |
| Cable Splitter — Coaxial1 | Cable Modem — Port 0 | Coaxial |
| Cable Splitter — Coaxial2 | TV — Port 0 | Coaxial |
| Cable Modem — Port 1 | Home Wireless Router — Internet | Copper straight-through |
| Office PC — FastEthernet0 | Home Wireless Router — GigabitEthernet 0/1 | Copper straight-through |
| Bedroom PC — FastEthernet0 | Home Wireless Router — GigabitEthernet 0/3 | Copper straight-through |
| Laptop wireless adapter | Home Wireless Router — 2.4 GHz WLAN | Wireless |

# 5. Addressing and Configuration Plan

The router's LAN configuration was visible during troubleshooting. The WAN fields initially showed 0.0.0.0 in the router GUI. The client devices were reported to have IP addresses and default gateways, and PC0 successfully received four replies when pinging its default gateway. The following table separates confirmed values from items that should be checked in the final file.

| Parameter | Value / status | Notes |
| --- | --- | --- |
| Router LAN IP address | 192.168.0.1 | Shown in the router's Setup / Network Setup screen. |
| LAN subnet mask | 255.255.255.0 | Shown in the router's Network Setup screen. |
| DHCP server | Enabled | Router distributes client IP configuration. |
| Starting DHCP address | 192.168.0.100 | Shown in the router's DHCP configuration. |
| Maximum number of users | 10 | Set according to the task instructions. |
| DHCP address range | 192.168.0.100–192.168.0.109 | Expected range from starting address and 10-user limit; verify final GUI. |
| 2.4 GHz SSID | MyHome | Configured wireless network name. |
| Wireless security | WPA2 Personal | Configured for the 2.4 GHz WLAN. |
| Router administrator username | admin | Username retained; default password was changed. |
| Router administrator password | admin | Password used in the lab; replace before any real deployment. |
| Wi-Fi passphrase | mypassphrase! | Passphrase used in the lab; replace before any real deployment. |
| Static DNS 1 | Initially 0.0.0.0; later manually changed | Final DNS IP was not recorded. |
| Router WAN IP / mask / gateway | Initially 0.0.0.0 | Final status not recorded; verify in the final project. |
| Web hostname | skillsforall.srv | Eventually displayed a Cisco Packet Tracer welcome page. |

Credential handling The report records the lab credentials because they were part of the simulation steps. Do not reuse these example credentials on a real router or wireless network. Change them to unique, strong values if adapting the design outside the lab.

# 6. Implementation Procedure

## 6.1 Coaxial connections

Selected Connections from the Network Components panel and chose the Coaxial cable.

Connected Cable Splitter Coaxial1 to Cable Modem Port 0.

Connected Cable Splitter Coaxial2 to TV Port 0.

Turned the TV status ON and checked for the expected TV-program image.

## 6.2 Ethernet connections

Selected Copper Straight-Through from Connections.

Connected Cable Modem Port 1 to the router's Internet port.

Connected Office PC FastEthernet0 to router GigabitEthernet 0/1.

Connected Bedroom PC FastEthernet0 to router GigabitEthernet 0/3.

Inspected the topology and observed green link indicators on the connected links.

## 6.3 Router access and basic configuration

Opened Office PC → Desktop → IP Configuration and selected DHCP.

Read the default gateway assigned to the Office PC and entered that address in the PC's Web Browser to open the router GUI.

Logged in using the activity's initial credentials.

Set Maximum Number of Users to 10 under Setup → Network Setup and saved the settings.

Changed and confirmed the administrator password under Administration, saved the settings, and logged in again.

## 6.4 Wireless configuration

Enabled the router's 2.4 GHz wireless radio.

Changed the SSID from Default to MyHome and saved the settings.

Selected WPA2 Personal under Wireless Security.

Entered the lab passphrase and saved the settings.

Opened Laptop → Desktop → PC Wireless, selected MyHome, entered the passphrase, and checked Link Information for a successful access-point connection.

## 6.5 Client IP configuration and website test

Configured the Bedroom PC to use DHCP and checked that it received a local IPv4 address beginning with 192.

Used the browser on the Laptop, Office PC, and Bedroom PC to request skillsforall.srv.

Advanced simulation time where needed to allow DHCP and other simulated network services to process.

# 7. Troubleshooting and Resolution

| Symptom / observation | Investigation | Action / result |
| --- | --- | --- |
| Browser showed “Host Name Unresolved” for skillsforall.srv. | The hostname could not be resolved even though client IPs and gateways were present. | Checked client addressing and continued with DNS troubleshooting. |
| PC successfully pinged its default gateway (4 replies received). | This confirmed local connectivity between the PC and the router at that time. | Focused next on name resolution and upstream/service configuration rather than rebuilding LAN links. |
| DNS Server field showed 0.0.0.0. | No usable DNS server was advertised in that field. | A DNS address was manually configured on the home router; the exact address was not recorded. |
| Router Internet IP, subnet mask, and default gateway showed 0.0.0.0 in a screenshot. | The router GUI did not show a valid upstream IPv4 lease at that moment. | Final WAN status was not recorded. This remains a verification item. |
| Hostname later opened a page headed “Welcome to Cisco Packet Tracer — Opening Opportunities, Mind Wide Open” with Quick Links. | The browser reached a page when using skillsforall.srv after a server was added during troubleshooting. | Hostname access was observed to work; confirm the DNS record points to the intended web server and test each client. |

Technical interpretation A successful ping to the default gateway proves local IP reachability to the router; it does not by itself prove WAN/internet access. Likewise, a browser page opening at a hostname demonstrates that the name resolved in the simulation, but the intended server IP, DNS record, and upstream connection should be checked to establish exactly what service was reached.

# 8. Testing and Verification

Use the checklist below to record the final state. Items marked “Observed” reflect the conversation record; other items should be verified in the saved Packet Tracer file.

| Test | Expected result | Recorded status |
| --- | --- | --- |
| Cable Splitter → Modem and TV | Both coaxial links connected; TV switched ON. | Configured per task; verify TV image. |
| Modem → Router Internet port | Copper link active; router receives upstream configuration if provided by the activity. | Link appeared connected; final WAN lease not recorded. |
| Office PC DHCP | Valid IPv4 address, subnet mask, and default gateway. | User reported IP and gateway present. |
| Office PC → Default Gateway ping | Replies received. | Observed: 4 replies received. |
| Router DHCP pool | Enabled; maximum users set to 10. | Configured; verify final page. |
| Wi-Fi SSID and security | MyHome visible; WPA2 Personal enabled. | Configured; laptop connection was attempted. |
| Laptop Wi-Fi association | Successful connection to access point. | User reported website page loaded at skillsforall.srv; verify Link Information if needed. |
| skillsforall.srv on a client | Browser displays the expected page. | Observed: Cisco Packet Tracer welcome page with Quick Links. |
| Office PC website test | Expected page loads. | Final per-device result not separately recorded. |
| Bedroom PC DHCP and website test | Valid DHCP address and expected page loads. | Final per-device result not separately recorded. |

# 9. Security Considerations

Change default router credentials before a real deployment.

Use WPA2-AES or WPA3 when supported by the device; use a unique, long Wi-Fi passphrase.

Keep router firmware updated on physical equipment.

Do not expose router administration to untrusted networks.

Use DHCP only for the intended client subnet and avoid overlapping address pools.

Use a trusted DNS service and verify that DNS records point to the intended servers.

Avoid publishing passwords or private addressing details in a public report if the network is reused outside a lab.

# 10. Results and Discussion

The project established the intended home-network structure using coaxial, Ethernet, and wireless connections. The router's LAN was configured with the address 192.168.0.1/24, DHCP was enabled with a ten-client limit, and a 2.4 GHz WLAN named MyHome was configured using WPA2 Personal. A wired client successfully pinged the default gateway, demonstrating local connectivity.

The initial browser failure was not caused simply by the physical link layout. The client DNS field showed 0.0.0.0, and the hostname could not be resolved. A DNS server was subsequently added and the router's DNS setting was manually changed. After this intervention, the browser displayed a page at skillsforall.srv. Since the final DNS IP, server address, and WAN lease were not captured, the report distinguishes the observed browser result from a fully verified end-to-end internet connection.

# 11. Conclusion and Future Improvements

This practical exercise demonstrated the workflow for building a small home network: identify devices, select suitable media, connect the correct ports, configure DHCP and Wi-Fi, secure administrative access, and test connectivity. Troubleshooting also showed that an IP address and default gateway are not sufficient for hostname-based browsing; DNS must be available and correctly configured. Local gateway reachability and upstream internet access are separate checks.

## Recommended improvements

Save the final Packet Tracer project file and capture screenshots of the topology, router DHCP settings, wireless security, DNS server record, and successful browser tests.

Record the final WAN IP address, gateway, and DNS server address.

Verify the DNS record for skillsforall.srv points to the actual web server used in the simulation.

Run the website test separately from the Laptop, Office PC, and Bedroom PC.

Add a simple IP-addressing table for each client based on the final values shown in Packet Tracer.

Use unique credentials and avoid keeping the lab's example password on real equipment.

# Appendix A — Final Configuration Checklist

☐  All required devices are present and labeled.

☐  Coaxial connections: splitter Coaxial1 → modem Port 0; splitter Coaxial2 → TV Port 0.

☐  Ethernet connections: modem Port 1 → router Internet; Office PC → GigabitEthernet 1; Bedroom PC → GigabitEthernet 2.

☐  TV status is ON and the expected program image is visible.

☐  Router LAN IP and subnet mask are recorded.

☐  DHCP is enabled and maximum users is set to 10.

☐  Router administrator password has been changed from the default.

☐  2.4 GHz radio is enabled; SSID is MyHome.

☐  Wireless security is WPA2 Personal and the laptop joins successfully.

☐  Each PC receives valid DHCP settings.

☐  DNS server IP is recorded and skillsforall.srv resolves to the intended server.

☐  Router WAN IP, subnet mask, and gateway are checked; any 0.0.0.0 values are investigated.

☐  Website loads from the Laptop, Office PC, and Bedroom PC.

☐  Packet Tracer project file and evidence screenshots are saved.

# Appendix B — Key Networking Concepts

| Term | Meaning |
| --- | --- |
| IP address | A logical address used to identify an interface on an IP network. |
| Subnet mask | Separates the network portion of an IPv4 address from the host portion. |
| Default gateway | The router a host uses to reach destinations outside its local subnet. |
| DHCP | Dynamic Host Configuration Protocol; automatically supplies IP configuration to clients. |
| DNS | Domain Name System; maps hostnames such as skillsforall.srv to IP addresses. |
| SSID | Service Set Identifier; the name of a wireless network. |
| WPA2 Personal | Wi-Fi security mode using a shared passphrase for network access. |
| WAN | Wide Area Network; in this design, the router's upstream-facing connection toward the provider. |
| LAN | Local Area Network; the home network connecting local wired and wireless devices. |
| Coaxial cable | Cable medium used here for the provider signal split between modem and television. |
| Copper straight-through | Ethernet cable used in the activity for modem/router and PC/router connections. |
| Ping | A diagnostic utility that tests IP reachability using ICMP echo messages. |

# References and Basis

Primary basis: the supplied Cisco Packet Tracer home-network activity instructions and the configuration/troubleshooting details reported during the practical session. No external sources were used. Values not visible or explicitly reported have been marked for verification rather than invented.